Major Tool & Machine, Inc.

Page: 1

1458 East 19th Street

MTM N/C: 19286

Date: 02/20/06

Indianapolis, IN 46218-4289

User ID: MCCORKLE

PRINCETON PLASMA PHYSICS LAB

Customer:

Telephone:

609-243-3655

Contact:

Mike Viola

mviola@pppl.gov

E-Mail:

609-243-3248

Fax:

S005243-F/Ln:7

Customer P.O.:

SE120-004-47 / SEAL RETAINER SCREW

Part:

Serial No./Qty:

Revision:

Drawing ID:

317-636-6433

Telephone:

DOUG MCCORKLE

Reported By:

Fax:

317-634-9420

E-Mail:

dMcCorkle@MajorTool.com

Problem:

seal retainer screw locked into nb cover after thermal cycling and  vacum testing. screw had no lubricant nor was it

silver plated.

fab was able to weld onto head and freely remove all 8 screws without damage to threads.

Proposed Disposition:

Customer disposition required.  The screws were destroyed during removal but the threaded holes are o.k.  The same screws located in the port 12 and 4 flanges were removed without incident (they were not subjected to high temp).

0

Number of additional pages:

Customer Disposition:       [   ] Use As Is          [  ] Rework          [  ] Repair          [  ] Scrap          [  ] Replace

Date:

Title:

Technical Contact Approval:

Date:

Title:

Buyer Approval:

Title:

Date:

Major Tool Implemented By:

/Open

n:\mtmapps\Mtnonc14.qrp

Major Tool and Machine, Inc.  1458 East 19th Street, Indianapolis, IN 46218-4289  Tel: 317-636-6433  Fax: 317-634-9420

Nonconformance Report: Major Tool NC19286

This is for SE120-004-47 / SEAL RETAINER SCREW

Problem:

seal retainer screw locked into nb cover after thermal cycling and  vacum testing. screw had no lubricant nor was it

silver plated.

fab was able to weld onto head and freely remove all 8 screws without damage to threads.

The screws were destroyed during removal but the threaded holes are o.k.

The same screws located in the port 12 and 4 flanges were removed without incident (they were not subjected to high temp).

Doug McCorkle

Project Disposition:

Approvals:

___________________________________________

Procurement Technical Representative

____________________________________________

Responsible Line Manager:

____________________________________________

Project Quality Assurance: