| Princeton Plasma Physics Laboratory Procedure |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ‘ Procedure Title: Field Period Assembly Component Receipt Inspection |  |  |  |  |  |  |
| Number: D-NCSX-FPA-QA1 |  | Revision: 0 |  |  | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| Procedure Approvals |  |  |  |  |  |  |
| Author: Mike Viola |  |  |  |  |  | Date: |
| ATI: John Edwards |  |  |  |  |  | Date: |
| RLM: Larry Dudek |  |  |  |  |  | Date: |
| Responsible Division: NCSX Project |  |  |  |  |  |  |
| Procedure Requirements Designated by RLM LABWIDE: |  |  |  |  |  |  |
| X | Work Planning Form (ENG-032) Add WP numbers |  |  | Lockout/Tagout (ESH-016) |  |  |
|  | Confined Space Permit (5008,SEC.8 Chap 5) |  |  | Lift Procedure (ENG-021) |  |  |
|  | Master Equip. List Mod (GEN-005) |  | X | ES&H Review (NEPA, IH, etc.) NEPA 1261 |  |  |
|  | RWP (HP-OP-20) |  |  | Independent Review |  |  |
|  | ATI Walkdown |  | X | Pre-Job Brief |  |  |
| X | Post-job Brief * |  |  |  |  |  |
| D-SITE SPECIFIC: |  |  |  |  |  |  |
| X | D-Site Work Permit (OP-AD-09) |  |  | Door Permit (OP-G-93) |  |  |
|  | Tritium Work Permit (OP-AD-49) |  |  | USQD (OP-AD-63) |  |  |
| X | Pre-Job Brief (OP-AD-79) |  |  | T-Mod (OP-AD-03) |  |  |
|  | ** DCA/DCN (OP-AD-104) # |  |  |  |  |  |

* Required for installations involving internal vacuum installations, critical lifts, and for the initial installation of repetitive work.

** 	OP-AD-104 was voided by procedure ENG-032. However, DCA’s that were open at the time of adoption of ENG-032 are still considered valid for work approval purposes.

REVIEWERS (designated by RLM) Rec’d/ Incorp. Comments

Accountable Technical Individual John W Edwards

Test Director

Independent Reviewer Hutch Neilson

D-Site Shift Supervisor

Independent

NCSX Dimensional Control Coordinator Brent Stratton

Vacuum

NCSX Field Supervisors Steve Raftopoulos, Mike Viola

Project Engineer for Stellerator Systems (WBS 1) Manager Brad Nelson (ORNL)

WBS Manager for Field Period Assembly Coils (WBS18) Dave Williamson (ORNL)

Quality Assurance/Quality Control. Judy Malsbury, J. Boscoe, Colin Phelps

Maintenance and Operations Division

Energy Conversion System/Motor Control Division

Engineering

Environmental Restoration & Waste Management Division

Environmental, Safety & Health Jerry Levine

Industrial Hygiene Bill Slavin

Health Physics

RLM Larry Dudek

| TRAINING (designated by RLM) |  |  |  |  |
| --- | --- | --- | --- | --- |
| No training required ______________ | Instructor: John Edwards |  |  |  |
| Personnel (group, job title or individual name) |  | Read Only | Instruction ### Pre-job Briefing | Hands On |
| Lead Tech. |  |  | X |  |
| Technicians performing task |  |  | X |  |
| Field Supervisors |  |  | X |  |
| Quality Control Representative |  |  | X |  |
| Training Rep. |  |  |  |  |
| RLM: Larry Dudek |  |  |  |  |

RECORD OF CHANGE

| Revision | Date | Description of Change |
| --- | --- | --- |
| 00 | XXXX | Initial release |

TABLE OF CONTENTS

1	Scope	1

2	Applicable Documents	1

2.1	NCSX-MIT/QA-185-01:	1

2.2	NCSX-PLAN-MFOP-00:	1

3	Safety Requirements:	1

3.1	Job Hazard Analysis:	1

4	Prerequisites & Conditions:	1

4.1	Pre-Job Briefing:	1

4.2	Daily Operations Startup and Shutdown:	1

4.3	Daily Startup Activities:	1

4.4	Daily Shutdown Activities:	2

5	Receipt and Inspection Process	2

5.1	General component Receipt and Inspection	2

5.2	Vacuum vessel Receipt and Inspection:	2

# Scope

The Field Period Assembly task involves assembling 3 identical 120° Field Period Assemblies using 5 separate workstations.  The field period assemblies include many small individual parts as well as major components being delivered by outside suppliers.

This procedure is used to describe the component receipt inspection in general and specifically addresses the Vacuum Vessel Sub Assemblies (VVSA) being delivered by Major Tool.

# Applicable Documents

## NCSX-MIT/QA-185-01:

All governing documents associated with this procedure, are identified in the MIT/QA Plan, document number NCSX-MIT/QA-185-01.

## NCSX-PLAN-MFOP-01:

All work processes are governed by the “NCSX Coil Manufacturing Facility Operations Plan”, document number NCSX-PLAN-MFOP-00.

# Safety Requirements:

All work will be performed in a safe manner in accordance with PPPL Environmental Safety & Health Directives ES&H 5008 and the “Integrated Safety Management” (ISM) policy.

## Job Hazard Analysis:

A JHA will be generated for each procedure, identifying existing or potential workplace hazards and to evaluate the risk of worker injury or illness associated with job tasks. (Reference document ESH-004 “Job Hazard Analysis”)  The IH representative will review the JHAs for accuracy as well as completeness.  It will be reviewed with all activity participants at the Pre-Job briefings.

# Prerequisites & Conditions:

## Pre-Job Briefing:

A pre-job briefing will be held, describing the processes and safety issues prior to starting any part of this procedure.  Attendance shall be documented via training sign-in sheet.  All procedural records will be made in Appendix A – Document Sign-off Record

See Appendix A – Document Sign-off Record

## Daily Operations Startup and Shutdown:

Each working day, it is required to complete and initial the daily operations startup log to ensure that the station is ready to commence work activities for the day.  The signoff log is located in the Daily Station Log.

## Daily Startup Activities:

### Check all daily supplies needed:

### Verify operation of all equipment needed that day.

### Check station for cleanliness

### Check that safety equipment needed for day’s activities are available

### Check that the day’s travelers and procedures are in their document holder.

### Once completed, date and initial daily log at the back of the Station Log Book.

## Daily Shutdown Activities:

### Turn off power to equipment not in use.

### Clean entire workstation area.

### Verify that all Traveler and data sheet information is complete.

### The Lead Technician shall verify that the Station’s Log Book has been completed and signed for the day.

### Once completed, date and initial daily log at the back of the Station Log Book.

# Receipt and Inspection Process

This procedure is to be used as guide to receive, inspect and store NCSX components.  Appendix B lists all material and/or equipment used with this procedure.  It shall serve as a running log of the items received their drawing, Requisition or Contract number, and storage number/location.

## General component Receipt and Inspection

### Perform Receipt Inspection

You should reference where all of the inspection criteria is located in the document.

#### Review requisition documents

#### Check items for obvious damage,

#### Check proper quantity,

#### Check labeling or marking,  [example: step 5.1

#### Check Material Certifications and file

#### Perform visual inspection for surface finish, weld quality, etc.

#### Perform permeability check, if applicable

#### Perform dimensional check, if applicable

Shouldn’t you provide some place to document the receipt inspection findings?

## Vacuum vessel Receipt and Inspection:

The VVSA, SE120-002, consists of a vessel shell referred to as a Vacuum Vessel Period Assembly (Period

Assembly), SE120-003, a Spacer Assembly (Spacer), SE121-014, two (2) Vacuum Vessel Blank Off Covers, SE121-102, two (2) Vacuum Vessel Seals, SE121-095, and the port extension assemblies with their associated blank flanges, seals, and fasteners. Three VVSA units, including all hardware in the referenced drawings, are to be procured, fabricated, and delivered by the Seller.

### Verify item receipt

NCSX-CSPEC-121-02 Paragraph

Your numbering of the steps appears incorrect.  Should this be labeled 5.1?

5.1 Labeling

Subassemblies and components, except bolts and standard hardware, shall be marked with unique serial numbers to provide positive identification. When such markings would impair proper functioning of the equipment, a metal, non-corrosive, non-magnetic tag shall be used. Match markings shall be provided to uniquely identify the location and positioning of all port extensions relative to the Period Assembly.

5.2 Packing and Skidding

All components shall be sealed, packaged, and skidded to provide protection against contamination, deterioration and damage during shipment. Vacuum sealing surfaces shall be protected from damage during shipping and handling. A plan shall be provided to the Laboratory prior to shipment which includes a description of methods to be used to preserve, package, skid, and identify equipment. The Seller shall contact the Laboratory ten days prior to shipment of the machine to confirm shipping method and route.

5.3 Marking

Each shipping skid shall be marked with the name of the Seller, Laboratory Purchase Order Number, the component name, and gross weight. Boxes containing loose parts, attachments, and accessories shall be marked identifying the assembly to which they belong, and where possible, boxes are to be secured to the skid of the unit.

Identify each item and list the item in the Material log

See Appendix A – Document Sign-off Record

### Perform Material Verification

NCSX-CSPEC-121-02 Paragraph

3.3.2.1 Sheet, Strip, and Plate

All as-supplied sheet, strip, and plate shall be annealed Alloy (UNS N06625) and meet the requirements of ASTM B 443.

3.3.2.2 Tubing and Piping

All Inconel tubing and pipe shall be seamless or welded Alloy (UNS N06625) and meet the requirements of ASTM B 444, or ASTM B 705. All austenitic stainless steel tubing shall be seamless or welded 316L alloy and meet the requirements of ASTM A 249/A 249 M-04A or ASTM A 213/A 213M-03.

3.3.2.3 Bar and Structural Shapes

All bar and structural shapes shall be annealed Alloy (UNS N06625) and meet the requirements of ASTM B 446.

3.3.2.4 Conflat Flanges

The conflat flanges shall meet the requirements of ASTM A 240.

3.3.2.5 Weld Filler Metal (Not Applicable)

3.3.2.6 Bolts

Conflat flange bolts shall be ASTM A 193, Grade B8; silver-plated, 12-point bolt kits provided with flanges from the flange manufacturer. Non-circular o-ring flange bolts, with the exception of the neutral beam port, shall use ASTM A453 Grade 660 bolts (A286) The neutral beam port, whose flanges are Inconel 625, shall use Inco 718 bolts per ASTM A1014.

3.3.2.7.1 Metal Seals

Seals for Conflat flanges shall use standard copper seals provided from the flange manufacturer.

3.3.2.7.2 Custom Flanges

Custom non-circular flanges, with the exception of the neutral beam port, will be sealed with two Viton A seals.

See Appendix A – Document Sign-off Record

### Perform visual Inspections

NCSX-CSPEC-121-02 Paragraph

3.2.2.1 Interior (Vacuum) Surfaces

Interior of the Period Assembly wall, Spacer, and port extensions shall be polished to a 32 micro-inch finish. Interior weld beads, scratches, and tooling marks resulting from fabrication shall be polished to a 32 micro-inch finish. Interior wall surface weld beads shall be ground to within .032 inch of the surface prior to polishing. Scratches, pits, weld pin holes and other surface imperfections exceeding depth limits set forth in the Engineering Drawings shall be repaired by welding before finish polishing.

3.2.3 Exterior Surface Finish

Mill finish on the exterior surfaces is acceptable, but any imperfections greater than 0.04 inches deep shall be weld repaired and ground smooth.

See Appendix A – Document Sign-off Record

### Perform Permeability check of welds components and shell surface

NCSX-CSPEC-121-02 Paragraph

Numbering of document steps is incorrect

4.2.3 Verification of Magnetic Permeability

To verify conformance to Section 3.2.4, magnetic permeability shall be measured in accordance with the requirements of ASTM A 800, Supplementary Requirement S1, but with the measurements taken in relative permeability, rather than ferrite content. All surfaces and features shall be checked with a calibrated Severn Permeability Indicator3 for compliance with Section 3.2.4. The surfaces of the VVSA components shall be checked and documented in a 6" x 6" grid. The weld seams in the shell wall, at the conflat flanges, and at the junction between the port extension, reinforcement, and shell shall be checked every 1/2" (both inside and outside surfaces wherever possible). Actual values need be recorded only for any out-of-tolerance conditions.

Where do you record out-of-tolerance findings?

3.2.4 Magnetic Permeability

Relative magnetic permeability of all components shall not exceed 1.02 except for welds (and heat affected zones) joining stainless steel to nickel chromium, which shall not exceed 1.2.

See Appendix A – Document Sign-off Record

### NOTE: It may be necessary to mount the VVSA into Station 1 per NCSX-FPA-001 in order to perform these next two steps.

### Verify tooling ball locations with respect to MTM data package. Perform metrology measurements with emphasis made to resolve impact of out of tolerance conditions.

Is this metrology step referenced in a metrology procedure?   How do you measure?

### Measure end flanges and compare with theoretical position.

See Appendix A – Document Sign-off Record

Appendix A - Document Sign-off Record

| TASK Description | Print Name | Initials | Date |
| --- | --- | --- | --- |
| 4.1 Pre job Briefing complete VVSA Field Supervisor |  |  |  |
| 5.2.1 Labeling and Marking Lead Tech |  |  |  |
| 5.2.2 Material Verification Lead Tech |  |  |  |
| 5.2.3 Surface Finish Lead Tech |  |  |  |
| 5.2.4 Permeability Lead Tech |  |  |  |
| 5.2.5 Metrology Tooling datums Metrology Supervisor |  |  |  |
| 5.2.6 End flange positions Metrology Supervisor |  |  |  |

Appendix B – Material Receipt List

| Inspector/ Date | General Description | Requisition/ Contract Number | Material | Reference Document/ Drawing | Documents | Damage | Quantity | Labeling | Material | Visual | Perm | Dimension | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VVSA | S-005243-F | Inconel | SE120-002 |  |  |  |  |  |  |  |  |  |