Proposed fix for VVSA #2 and #3 end flanges

There are two conditions of concern from a VV assy viewpoint:

- shell outer surface is outside flange od, or

- shell inner surface is inside of shell id

- shell outer surface is outside flange od:  MTM to do the following

- Make inside weld between flange and shell and clean up

- Weld leak check cover plate on

- Finish shell to flange weld on outside and butter up od of flange to match or exceed outside surface of shell

- Perform leak check

- Remove leak check cover plate

- Clean up face surface of flanges

- 3/8 minimum weld throat, smoothed out and blended for vacuum service on inside

- Shell inner surface is inside of shell id:

- PPPL will butter up id of flange by carrying assembly weld over to shell