Major Tool & Machine, Inc.

Page: 1

1458 East 19th Street

MTM N/C: 19116

Date: 02/27/06

Indianapolis, IN 46218-4289

User ID: MCCORKLE

PRINCETON PLASMA PHYSICS LAB

Customer:

Telephone:

609-243-3655

Contact:

Mike Viola

mviola@pppl.gov

E-Mail:

609-243-3248

Fax:

S005243-F/Ln:7

Customer P.O.:

/

Part:

(all std. CF blanks)

Qty:

2

Revision:

SE120-004

Drawing ID:

317-636-6433

Telephone:

DOUG MCCORKLE

Reported By:

Fax:

317-634-9420

E-Mail:

dMcCorkle@MajorTool.com

Problem:

IT WAS REQUIRED TO OVERSIZE HOLE PATTERN  ON MAJORITY OF ALL ROUND BLANK COVERS

.015 DIAMETER  FROM ORIGINAL HOLE SIZE TO ALLOW FOR PROPER BOLT CLEARANCE. ONLY

PARTIAL QTY ON LOT 7 OR 1 HAS BEEN OVERSIZED AT THIS POINT.

Proposed Disposition:

In order to provide adequate clearance for assembling the MDC covers (original specification) to the Trinos flanges

(replacements for the original specified MDC flanges), it was necessary to machine the bolt holes of the flanges

oversize by approximately 0.010-0.020".  Also, the O.D. of the standard copper seals obtained from MDC had to be

turned smaller by approximately 0.03-0.04" (on diameter).  Propose “use as is” disposition.

NC Type:  Notification Only.

Number of additional pages:

Customer Disposition:       [   ] Use As Is          [  ] Rework          [  ] Repair          [  ] Scrap          [  ] Replace

Date:

Title:

Technical Contact Approval:

Date:

Title:

Buyer Approval:

Title:

Date:

Major Tool Implemented By:

/Open

n:\mtmapps\Mtnonc14.qrp

Major Tool and Machine, Inc.  1458 East 19th Street, Indianapolis, IN 46218-4289  Tel: 317-636-6433  Fax: 317-634-9420

Nonconformance Report: Major Tool NC19116

This is for SE120-004

Problem:

IT WAS REQUIRED TO OVERSIZE HOLE PATTERN  ON MAJORITY OF ALL ROUND BLANK COVERS

.015 DIAMETER  FROM ORIGINAL HOLE SIZE TO ALLOW FOR PROPER BOLT CLEARANCE. ONLY

PARTIAL QTY ON LOT 7 OR 1 HAS BEEN OVERSIZED AT THIS POINT.

In order to provide adequate clearance for assembling the MDC covers (original specification) to the Trinos flanges

(Replacements for the original specified MDC flanges), it was necessary to machine the bolt holes of the flanges

oversize by approximately 0.010-0.020".  Also, the O.D. of the standard copper seals obtained from MDC had to be

turned smaller by approximately 0.03-0.04" (on diameter).  Propose “use as is” disposition.

NC Type:  Notification Only.

Doug McCorkle

Project Disposition:

Use as is.

Additional remarks:

Does this Change Impact Material Already Procured or Parts/Assemblies Already Assembled/Manufactured using this Material:    Yes    No

If “Yes”, what is the recommended disposition of this material/part/assembly?

Return rejected MDC flanges to PPPL with the first VVSA

Approvals:

___________________________________________

Procurement Technical Representative

____________________________________________

Responsible Line Manager:

____________________________________________

Project Quality Assurance: