Major Tool & Machine, Inc.

Page: 1

1458 East 19th Street

MTM N/C: 19254

Date: 02/14/06

Indianapolis, IN 46218-4289

User ID: MCCORKLE

PRINCETON PLASMA PHYSICS LAB

Customer:

Telephone:

609-243-3655

Contact:

Mike Viola

mviola@pppl.gov

E-Mail:

609-243-3248

Fax:

S005243-F/Ln:8

Customer P.O.:

SE120-004-52 / O-RING, METAL, HELICOFLEX TYPE

Part:

2 sets

Qty:

1

Revision:

SE120-004

Drawing ID:

317-636-6433

Telephone:

DOUG MCCORKLE

Reported By:

Fax:

317-634-9420

E-Mail:

dMcCorkle@MajorTool.com

Problem:

BOTH LOT 8 AND 9 OUTER HELICOFLEX METAL SEALS ARE TOO SMALL TO ALLOW FOR PROPER

FIT OF RETAINING STRIP. ACTUAL AMOUNT TOO SMALL IS UNKNOWN,VISUALLY APPEARS TO BE

.120 TOO SMALL. IN OTHER WORDS THE OD OF SEAL RETAINER WOULD NEED .100 OFF ALL WAY

AROUND TO MAKE WORK.

LOT 9 HAS MINOR DAMAGE TO SEALING FACE ON CUSTOMER SUPPLIED SEAL.

Proposed Disposition:

CUSTOMER DISPOSITION REQUIRED

Number of additional pages:

Customer Disposition:       [   ] Use As Is          [  ] Rework          [  ] Repair          [  ] Scrap          [  ] Replace

Date:

Title:

Technical Contact Approval:

Date:

Title:

Buyer Approval:

Title:

Date:

Major Tool Implemented By:

/Open

n:\mtmapps\Mtnonc14.qrp

Major Tool and Machine, Inc.  1458 East 19th Street, Indianapolis, IN 46218-4289  Tel: 317-636-6433  Fax: 317-634-9420

Nonconformance Report: Major Tool NC19254

This is for SE120-004-52 / O-RING, METAL, HELICOFLEX TYPE

Problem:

BOTH LOT 8 AND 9 OUTER HELICOFLEX METAL SEALS ARE TOO SMALL TO ALLOW FOR PROPER

FIT OF RETAINING STRIP. ACTUAL AMOUNT TOO SMALL IS UNKNOWN, VISUALLY APPEARS TO BE

.120 TOO SMALL. IN OTHER WORDS THE OD OF SEAL RETAINER WOULD NEED .100 OFF ALL WAY

AROUND TO MAKE WORK.

LOT 9 HAS MINOR DAMAGE TO SEALING FACE ON CUSTOMER SUPPLIED SEAL.

Doug McCorkle

Project Disposition:

Rework first retainer set as desired - retainer strip radii may be cut off to allow for circumferential difference or tongue depressors may be used in place of seal retainer to hold seals in place during assembly.

Second and third set may be left per print.

Approvals:

___________________________________________

Procurement Technical Representative

____________________________________________

Responsible Line Manager:

____________________________________________

Project Quality Assurance: