Major Tool & Machine, Inc.

Page: 1

1458 East 19th Street

MTM N/C: 19289

Date: 02/20/06

Indianapolis, IN 46218-4289

User ID: MCCORKLE

PRINCETON PLASMA PHYSICS LAB

Customer:

Telephone:

609-243-3655

Contact:

Mike Viola

mviola@pppl.gov

E-Mail:

609-243-3248

Fax:

S005243-F/Ln:2

Customer P.O.:

/

Part:

3

Qty:

0

Revision:

SE120-003

Drawing ID:

317-636-6433

Telephone:

DOUG MCCORKLE

Reported By:

Fax:

317-634-9420

E-Mail:

dMcCorkle@MajorTool.com

Problem:

PORT NB HAS DEFORMATION TO HOLE PATTERN AFTER WELDING HAS BEEN COMPLETED ON

PORT 4  A/B AND 12 A/B

HOLES DO NOT ALIGN TO MATING THREADED HOLE IN NB COVER. HOLES DID ALIGN AFTER NB

WAS WELDED IN,BUT DO NOT NOW THAT PORT 4 AND 12 IS COMPLETED.

Proposed Disposition:

CUSTOMER DISPOSITION REQUIRED.

THE FLANGE IS ALSO OUT OF FLAT UP TO 1/16".

UNIT # 1 HOLES WERE OPENED UP TO 5/8" DIAMETER TO PROVIDE THE NECESSARY CLEARANCE

FOR ASSEMBLY PRIOR TO VACUUM TESTING.  THE EXTRA WELDING ON UNIT # 1 WAS SUSPECTED

TO BE THE CAUSE OF THE EXCESS DISTORTION.  THIS PROVED INCORRECT AFTER UNIT # 2

FLANGE ALSO DISTORTED.  UNIT # 3 HAS NOT BEEN INSTALLED YET, BUT IS EXPECTED TO REACT

THE SAME.  MTM RECOMMENDS OPENING ALL NB FLANGE HOLES UP TO 5/8" AND RE-FACING THE

FLANGE FLAT AFTER ALL PORTS ARE WELDED IN PLACE (A MINIMUM THICKNESS WILL BE

NEEDED).

0

Number of additional pages:

Customer Disposition:       [   ] Use As Is          [  ] Rework          [  ] Repair          [  ] Scrap          [  ] Replace

Date:

Title:

Technical Contact Approval:

Date:

Title:

Buyer Approval:

Title:

Date:

Major Tool Implemented By:

/Open

n:\mtmapps\Mtnonc14.qrp

Major Tool and Machine, Inc.  1458 East 19th Street, Indianapolis, IN 46218-4289  Tel: 317-636-6433  Fax: 317-634-9420

Nonconformance Report: Major Tool NC19289

This is for SE120-003

Problem:

PORT NB HAS DEFORMATION TO HOLE PATTERN AFTER WELDING HAS BEEN COMPLETED ON

PORT 4  A/B AND 12 A/B

HOLES DO NOT ALIGN TO MATING THREADED HOLE IN NB COVER. HOLES DID ALIGN AFTER NB

WAS WELDED IN,BUT DO NOT NOW THAT PORT 4 AND 12 IS COMPLETED.

THE FLANGE IS ALSO OUT OF FLAT UP TO 1/16".

UNIT # 1 HOLES WERE OPENED UP TO 5/8" DIAMETER TO PROVIDE THE NECESSARY CLEARANCE

FOR ASSEMBLY PRIOR TO VACUUM TESTING.  THE EXTRA WELDING ON UNIT # 1 WAS SUSPECTED

TO BE THE CAUSE OF THE EXCESS DISTORTION.  THIS PROVED INCORRECT AFTER UNIT # 2

FLANGE ALSO DISTORTED.  UNIT # 3 HAS NOT BEEN INSTALLED YET, BUT IS EXPECTED TO REACT

THE SAME.  MTM RECOMMENDS OPENING ALL NB FLANGE HOLES UP TO 5/8" AND RE-FACING THE

FLANGE FLAT AFTER ALL PORTS ARE WELDED IN PLACE (A MINIMUM THICKNESS WILL BE

NEEDED).

Doug McCorkle

Project Disposition:

Rework as proposed – Out of flat flanges are not acceptable.  The project agrees that the flanges should be faced off after welding.  Since hole pattern errors would require custom mating parts in the future, the project accepts the proposal to open up the holes to 5/8 diameter.

Approvals:

___________________________________________

Procurement Technical Representative

____________________________________________

Responsible Line Manager:

____________________________________________

Project Quality Assurance: