Major Tool & Machine, Inc.

Page: 1

1458 East 19th Street

MTM N/C: 19391

Date: 03/10/06

Indianapolis, IN 46218-4289

User ID: MCCORKLE

PRINCETON PLASMA PHYSICS LAB

Customer:

Telephone:

609-243-2441

Contact:

LARRY SUTTON

S-04286-F

E-Mail:

609-243-2021

Fax:

S005243-F/Ln:1

Customer P.O.:

/

Part:

Serial No./Qty:

2

Revision:

SE120-004

Drawing ID:

317-636-6433

Telephone:

DOUG MCCORKLE

Reported By:

Fax:

317-634-9420

E-Mail:

dMcCorkle@MajorTool.com

Problem:

The .637 +/-.005 dimension varies around part over and undersized, .480-.9

Proposed Disposition:

SUBMITTING TO PPPL.

REFERENCE DRAWING SE121-013 ZONE G4.

Number of additional pages:

Customer Disposition:       [   ] Use As Is          [  ] Rework          [  ] Repair          [  ] Scrap          [  ] Replace

Date:

Title:

Technical Contact Approval:

Date:

Title:

Buyer Approval:

Title:

Date:

Major Tool Implemented By:

/Closed /WO:65678-1

n:\mtmapps\Mtnonc14.qrp

Major Tool and Machine, Inc.  1458 East 19th Street, Indianapolis, IN 46218-4289  Tel: 317-636-6433  Fax: 317-634-9420

Nonconformance Report: Major Tool NC19391

This is for REFERENCE DRAWING SE121-013 ZONE G4.

Problem:

The .637 +/-.005 dimension varies around part over and undersized, .480-.9

Doug McCorkle

Project Disposition:

Undersize condition rejected – Flange seal groove cannot be less than .62 -.005” per model.  For the undersize condition, ID of seal must be ground back and seal groove filled or additional material added to flange ID to restore the .62” dimension. Oversize condition (0.9”) may be tolerated if acceptable corrective action allows seal to be welded to flange.”  Please propose corrective actions for both conditions for approval.

Approvals:

___________________________________________

Procurement Technical Representative

____________________________________________

Responsible Line Manager:

____________________________________________

Project Quality Assurance: