Major Tool & Machine, Inc.

Page: 1

1458 East 19th Street

MTM N/C: 19392

Date: 03/10/06

Indianapolis, IN 46218-4289

User ID: MCCORKLE

PRINCETON PLASMA PHYSICS LAB

Customer:

Telephone:

609-243-2441

Contact:

LARRY SUTTON

S-04286-F

E-Mail:

609-243-2021

Fax:

S005243-F/Ln:1

Customer P.O.:

/

Part:

Serial No./Qty:

2

Revision:

SE120-004

Drawing ID:

317-636-6433

Telephone:

DOUG MCCORKLE

Reported By:

Fax:

317-634-9420

E-Mail:

dMcCorkle@MajorTool.com

Problem:

The .469 +/-.005 dimension varies around part over and undersized, .240-.432

Proposed Disposition:

SUBMITTING TO PPPL.

REFERENCE DRAWING SE121-013, ZONE B5

Number of additional pages:

Customer Disposition:       [   ] Use As Is          [  ] Rework          [  ] Repair          [  ] Scrap          [  ] Replace

Date:

Title:

Technical Contact Approval:

Date:

Title:

Buyer Approval:

Title:

Date:

Major Tool Implemented By:

/Closed /WO:65678-1

n:\mtmapps\Mtnonc14.qrp

Major Tool and Machine, Inc.  1458 East 19th Street, Indianapolis, IN 46218-4289  Tel: 317-636-6433  Fax: 317-634-9420

Nonconformance Report: Major Tool NC19392

This is for REFERENCE DRAWING SE121-013, ZONE B5

Problem:

The .469 +/-.005 dimension varies around part over and undersized, .240-.432

Doug McCorkle

Project Disposition:

It appears from these dimensions that the tapped holes would break through the seal groove.  Fill all holes to provide a minimum 1/8” cap weld and grind flush for all flanges.  Advise PPPL method of filling holes for approval prior to work.  Solution must avoid virtual leaks.  Inspect with 8x magnification to verify complete seal.

Approvals:

___________________________________________

Procurement Technical Representative

____________________________________________

Responsible Line Manager:

____________________________________________

Project Quality Assurance: