Major Tool & Machine, Inc.

Page: 1

1458 East 19th Street

MTM N/C: 19868

Date: 06/08/06

Indianapolis, IN 46218-4289

User ID: MCCORKLE

PRINCETON PLASMA PHYSICS LAB

Customer:

Telephone:

609-243-3655

Contact:

Mike Viola

mviola@pppl.gov

E-Mail:

609-243-2021

Fax:

S005243-F/Ln:2

Customer P.O.:

/

Part:

1

Qty:

0

Revision:

SE120-003

Drawing ID:

317-636-6433

Telephone:

DOUG MCCORKLE

Reported By:

Fax:

317-634-9420

E-Mail:

dMcCorkle@MajorTool.com

Problem:

PORT NB HAS DEFORMATION TO HOLE PATTERN AFTER WELDING HAS BEEN COMPLETED ON

PORT 4  A/B AND 12 A/B

HOLES DO NOT ALIGN TO MATING THREADED HOLE IN NB COVER. HOLES DID ALIGN AFTER NB

WAS WELDED IN,BUT DO NOT NOW THAT PORT 4 AND 12 IS COMPLETED.

ALSO, THE FACE OF PORT NB HAS GONE OUT OF FLAT BY APPROXIMATELY 1/16".

Proposed Disposition:

This issue was originally grouped (all three lots) within NCR 19289.

The creation of 19868 and 19869 separates each lot for individual attention.

Recommend facing the port flange face flat and boring the holes to 5/8" diameter to provide necessary clearance.

0

Number of additional pages:

Customer Disposition:       [   ] Use As Is          [  ] Rework          [  ] Repair          [  ] Scrap          [  ] Replace

Date:

Title:

Technical Contact Approval:

Date:

Title:

Buyer Approval:

Title:

Date:

Major Tool Implemented By:

/Open

n:\mtmapps\Mtnonc14.qrp

Major Tool and Machine, Inc.  1458 East 19th Street, Indianapolis, IN 46218-4289  Tel: 317-636-6433  Fax: 317-634-9420

Nonconformance Report: Major Tool NC19868

This is for:  SE120-003  VVSA # 2 NB port and flange

Problem:

PORT NB HAS DEFORMATION TO HOLE PATTERN AFTER WELDING HAS BEEN COMPLETED ON

PORT 4  A/B AND 12 A/B

HOLES DO NOT ALIGN TO MATING THREADED HOLE IN NB COVER. HOLES DID ALIGN AFTER NB

WAS WELDED IN,BUT DO NOT NOW THAT PORT 4 AND 12 IS COMPLETED.

ALSO, THE FACE OF PORT NB HAS GONE OUT OF FLAT BY APPROXIMATELY 1/16".

Major Tool recommended Disposition:

This issue was originally grouped (all three lots) within NCR 19289.

The creation of 19868 and 19869 separates each lot for individual attention.

Recommend facing the port flange face flat (not to exceed 1/16”) and boring the holes to 5/8" diameter to provide necessary clearance.

Project Disposition:

Proceed as recommended:

“Recommend facing the port flange face flat (not to exceed 1/16”) and boring the holes to 5/8" diameter to provide necessary clearance.”

Approvals:

___________________________________________

Procurement Technical Representative

____________________________________________

Responsible Line Manager: