Major Tool & Machine, Inc.

Page: 1

1458 East 19th Street

MTM N/C: 19869

Date: 08/15/06

Indianapolis, IN 46218-4289

User ID: MCCORKLE

PRINCETON PLASMA PHYSICS LAB

Customer:

Telephone:

609-243-3655

Contact:

Mike Viola

mviola@pppl.gov

E-Mail:

609-243-2021

Fax:

S005243-F/Ln:3

Customer P.O.:

/

Part:

1 (VVSA 3)

Serial No./Qty:

1

Revision:

SE122-072

Drawing ID:

317-636-6433

Telephone:

DOUG MCCORKLE

Reported By:

Fax:

317-634-9420

E-Mail:

dMcCorkle@MajorTool.com

Problem:

PORT NB HAS DEFORMATION TO HOLE PATTERN AFTER WELDING HAS BEEN COMPLETED ON

PORT 4  A/B AND 12 A/B

HOLES DO NOT ALIGN TO MATING THREADED HOLE IN NB COVER. HOLES DID ALIGN AFTER NB

WAS WELDED IN,BUT DO NOT NOW THAT PORT 4 AND 12 IS COMPLETED.

Proposed Disposition:

The flange has been reworked in the same manner as unit 1 & 2.  The NB flange face was re-cut to restore flatness.

The holes were opened up to 5/8” dia.  The flange thickness now checks 1.228-1.241.

0

Number of additional pages:

Customer Disposition:       [   ] Use As Is          [  ] Rework          [  ] Repair          [  ] Scrap          [  ] Replace

Date:

Title:

Technical Contact Approval:

Date:

Title:

Buyer Approval:

Title:

Date:

Major Tool Implemented By:

/Open

n:\mtmapps\Mtnonc14.qrp

Major Tool and Machine, Inc.  1458 East 19th Street, Indianapolis, IN 46218-4289  Tel: 317-636-6433  Fax: 317-634-9420

Nonconformance Report: Major Tool NC19869

This is for:  VVSA # 3 Profile SE122-072

Problem:

PORT NB HAS DEFORMATION TO HOLE PATTERN AFTER WELDING HAS BEEN COMPLETED ON

PORT 4 A/B AND 12 A/B

HOLES DO NOT ALIGN TO MATING THREADED HOLE IN NB COVER. HOLES DID ALIGN AFTER NB

WAS WELDED IN, BUT DO NOT NOW THAT PORT 4 AND 12 IS COMPLETED.

MTM Recommended Disposition:

The flange has been reworked in the same manner as unit 1 & 2.  The NB flange face was re-cut to restore flatness.

The holes were opened up to 5/8” dia.  The flange thickness now checks 1.228-1.241.

Use as is.

Project Disposition:

Use as is.

Approvals:

___________________________________________

Procurement Technical Representative

____________________________________________

Responsible Line Manager: