| NCSX RFD Part I | Number: RFD-12-019 |  | RFD Description: Weld Prep on port extensions |
| --- | --- | --- | --- |
| Initiator: Doug McCorkle |  | Organization: Major Tool & Machine, Inc. |  |
| List of Impacted Documents: (Specification, MIT/QA Plan, SOW, drawing, etc.) SE120-003 |  |  |  |
| Cost Impact: (If none, so state) None |  |  |  |
| Schedule Impact: (If none, so state) None |  |  |  |
| Quality Impact: (If none, so state) None |  |  |  |
| State Requirement Deviation is Requested For: (Specification, MIT/QA Plan, SOW, drawing, etc.): |  |  |  |
| Full Description of the Deviation Requested: (Use continuation pages, e-mails, letter, sketches, etc. as needed and include amplifying information as appropriate to support deviation request.) Deviation Requested: Add NOTE to SE120-003 Sheet 3: Weld prep detail for Port 4 shall be bevel the stub only 30-40 degrees and keep the port extension square. Weld prep detail for the Dome VVSA Lot 1 shall be a 60 included angle v-groove and Lot 2 and 3 shall be a 40 degree included angle v-groove (with an approximate 1/16" root opening). Justification: Required for proper function. Provides easier access and enough width at the face to easily get full penetration and not provide an excessive amount of inter-passes to fill. |  |  |  |
| Attachments: None |  |  |  |
| Initiator Signature: Douglas J. McCorkle Date: |  |  |  |

| NCSX RFD Part II | Number: 12-019 |  | RFD Description: Weld Prep on port extensions |
| --- | --- | --- | --- |
| RLM: Brad Nelson |  | Organization: PPPL |  |
| Impact on Interfaces with Other WBS Elements/Items: (If none, so state): NONE |  |  |  |
| RLM Recommendation: Approve Do Not Approve Additional remarks: Result of discussion with Paul Goranson and Major Tool. Drawing will NOT be changed but a stamp will be added referring to this RFD. |  |  |  |
| RLM Signature: |  |  |  |
| Project Disposition: Approved. No ECP required. NCSX Systems Engineering Support Manager Approved. ECP will be assigned and processed. Not Approved. Reason(s) for disapproval: |  |  |  |