| NCSX RFD Part I | Number: NCSX-RFD-12-003 |  | RFD Description: VVSA Tube Clip Thickness |
| --- | --- | --- | --- |
| Initiator: Doug McCorkle |  | Organization: Major Tool |  |
| List of Impacted Documents: SE120-005 |  |  |  |
| Cost Impact: (If none, so state) None |  |  |  |
| Schedule Impact: (If none, so state) None |  |  |  |
| Impact on Interfaces with Other WBS Elements/Items: (If none, so state) None |  |  |  |
| Full Description of the Deviation Requested: (Use continuation pages, e-mails, letter, sketches, etc. as needed and include amplifying information as appropriate to support deviation request.) JUSTIFICATION: Please look at drawing SE120-005, sheet 2, zone E2. Detail 39 (Tube Clip) is dimensioned as 28 ga and references a tolerance of (.020-.010). The parts list on sheet 1 specifies material grade UNS NO6625. Product Specification NCSX CSPEC-121-02-03 3.3.2.1 requires sheet, strip, and plate to conform to ASTM B443. ASTM B443 does not provide a thickness tolerance for plate less than 0.018-0.025" within the specification. With this in mind, we are moving forward with the plan of using material in the range of 0.010-0.020" for the Tube Clips. DEVIATION REQUEST: Change SE120-005, sheet 2, zone E2 detail 39 (Tube Clip) thickness tolerance to .008”-.025” |  |  |  |
| Attachments: |  |  |  |
| Initiator Signature: Date: |  |  |  |