| NCSX RFD Part I | Number: 12-015 |  | RFD Description: NCSX VVSA Spacer Machining |
| --- | --- | --- | --- |
| Initiator: Doug McCorkle |  | Organization: Major Tool and Machine |  |
| List of Impacted Documents: (Specification, MIT/QA Plan, SOW, drawing, etc.) NCSX-CSPEC-121-02 SE121-014 |  |  |  |
| Cost Impact: (If none, so state) NONE |  |  |  |
| Schedule Impact: (If none, so state: NONE |  |  |  |
| Quality Impact: (If none, so state): NONE |  |  |  |
| State Requirement Deviation is Requested For: (Specification, MIT/QA Plan, SOW, drawing, etc.) SE121-014 Reference dimension 6.50 and 1.00 |  |  |  |
| Full Description of the Deviation Requested: (Use continuation pages, e-mails, letter, sketches, etc. as needed and include amplifying information as appropriate to support deviation request.) Reference drawing SE121-014. The dimension controlling the overall height (6.50) is interpreted as a reference dimension by MTM. It is based on the flanges being 1.0” thick (also reference dimensions). MTM manufactured the flange details using thicker material providing excess stock allowance for final machining to this height after welding. This approach is necessary to maintain the 0.015” flatness requirement. The Spacer final machining operation is planned to commence soon. It has been discussed that PPPL may benefit by leaving this excess stock on the faces for additional flexibility if needed during final fit-up. If this option is desired by PPPL, the 6.50 and 1.00 reference dimensions would not be compliant by MTM. |  |  |  |
| Attachments: N/A |  |  |  |
| Initiator Signature: Doug McCorkle Date: 14Feb2006 |  |  |  |

| NCSX RFD Part II | Number: 12-015 |  | RFD Description: NCSX VVSA Spacer Machining |
| --- | --- | --- | --- |
| RLM: Brad Nelson |  | Organization: PPPL |  |
| Impact on Interfaces with Other WBS Elements/Items: (If none, so state): NONE |  |  |  |
| RLM Recommendation: Approve Do Not Approve Additional remarks: Machine minimum amount of stock to ensure that carbon steel weld metal is removed from cover plate using a permeability gage. Machine flat and parallel. |  |  |  |
| RLM Signature: |  |  |  |
| Project Disposition: Approved. No ECP required. Approved. ECP will be assigned and processed. NCSX Systems Engineering Support Manager Not Approved. Reason(s) for disapproval: |  |  |  |