Major Tool:  cold forming

- generally very good.

- why is price so high, if they think they can meet eventual cost target of $3M ?

- why is cost so high, given that they thought it was so similar to current activities?

- Statement on page 9:  milestones can be met only if work starts no later than end of Jan 2003 !!  What are implications?

- Are we willing for them to shift region for PVVS, as suggested, to allow reuse of dies?  Should this be offered to other potential venders?

EIO:  warm forming

- page 6, suggestion of mechanical polish instead of electropolish:  did the spec require electro-polish?  Do they realize that they serve different purposes?

- Is port assembly suggestion acceptable?

- what does accuracy comment on page 6 mean??

- lots of shipping around

- high cost for project coordination and reporting.  Generally high costs for 3.1.x & 3.3.x

AmEx:  explosive

- should check with Dave Anderson on what he thinks about these guys

- how many pieces do they plan to make it out of?

- why laser cutting instead of water jet?

- anyone know the NDE company?  Where is their QA documentation?

- interesting that first step, page 7 of section 2, is to review and approve welding procedures and audit welding shop

- note on page 9 of section 2, on importance of wrought inconel instead of ??

Rohwedder: ??

- Have not identified forming process!!?

- page 33 appears to imply that they would select an unknown sub for forming the shell components!

- not clear that they understand the complexity…?

DC Fabricators:  thick cold form

- forming plates 1.5” thick, machining to shape, then welding !!

- electropolishing happens before welding!  What cleans up weld?

- stiffeners added to control weld distortion (page 8).  When are they removed?

Amer Ind. Tech.

- I only see costing of PVVS fabrication.  What about other tasks?

- how many pieces will the vessel be made from?

- surface finish?

- diagram on page 21 has PPPL standing between Brenner and Amer Ind.  Curious.

DWE

- Appears to be fixed cost proposal!

- cost breakdown?

- Concern about tolerances.  Page 6

- concern that the nozzle assembly process is not practical.  Do we agree?

- strong preference expressed for laser tracker.  Implications for vendors doing without?

- Non fabrication tasks are ~ ½ the cost !!  324.8 M euros

- lost of pieces, lots of welding.