NCSX

Design Basis Analysis

NCSX Vacuum Vessel Heat Balance Analysis

NCSX-CALC-12-003-00

### Draft A

12 March 2004

Prepared by:

_______________________________________

P. L. Goranson, ORNL

I have reviewed this calculation and, to my professional satisfaction, it is properly performed and correct.  I concur with analysis methodology and inputs and with the reasonableness of the results and their interpretation.

Reviewed by:

_______________________________________

F. Dahlgren, PPPL Engineer

Controlled Document

THIS IS AN UNCONTROLLED DOCUMENT ONCE PRINTED.

Check the NCSX Engineering Web prior to use to assure that this document is current.

OAK RIDGE NATIONAL LABORATORY:		NCSX-CALC-123-03-00_dA

Plasma Technology and Applications

To: W. Reiersen							DATE: 12 Mar 2004

FROM: P. Goranson	SUBJECT:	NCSX Vacuum Vessel Heat Balance Analysis

Introduction

The NCSX vacuum vessel utilizes gas tracing tubes on the exterior vessel wall and electrical resistance heaters on the port extension walls to provide heating during bakeout and standby operation of the vessel.  Insulation is provided to limit the heat losses from the vessel system to the cryostat and modular coils, both of which are maintained at cryogenic temperature.  Analyses were performed to determine the heat losses through the insulation and heat inputs required to maintain steady state thermal equilibrium.  The results of these analyses are being used to determine the adequacy of the vessel tracing and electrical heaters and to determine their operating parameters.

Performance Requirements and Criteria

The NCSX General Requirements Document, NCSX-GRD-00 requires that:

- The vacuum vessel and interior components must be baked at 350 C and maintained at 20-40 C before and after operational shots.

- During bakeout, the port extensions are to be maintained at 150 C at the flanges ends and  350 C at vessel end, with gradient between.

- The cryostat and modular coils are maintained at 80 K during both bakeout and standby operation.

Methodology

The analysis was done as a spreadsheet in MICROSOFT EXCEL representing the vessel and ports as simple areas conducting heat across insulating layers to constant temperature heat sinks.  Input variables to the spreadsheet were insulation thicknesses, conductivity, vessel temperatures, and surface areas.  The outputs were the heat loss from the vessel body and port extensions to the cryostat and coil bodies.  The port heating/cooling balance was done with finite differences, iterating along the port length until the boundary conditions [temperatures] were met.  The output was the net loss and heat in.  The values for a typical port were converted to surface fluxes which were used as typical values to estimate the totals for all ports.

Assumptions

Temperatures(K)

- Crostat & Coil 		80

- Vessel bake 		623

- Cryostat Exterior  		288

- Vessel idle   		293

- Ambient   			293

Dimensions

- Cryostat is 123" radius by 119" tall.

- Vessel area, excluding ports is 52200 in2.

- Port standpipe area is 65000 in2.

- Coil area facing vessel  is 23400 in2.

Material Properties

Insulation conductivity  = 0.0002 W/cmK

N2 heat vap. J/g at 77K = 197.6

Insulation thicknesses

An efficiency factor of 75% is assumed.

Vessel microtherm

Cryo foam

Backfill insul.

Port insul.

Path to coil

Port cover

thickness(cm)

thickness(cm)

effec. thick.(cm)

thickness(cm)

(cm)

(cm)

Bake

1.27

20.0

15.0

2.54

8.00

1.27

2

20.0

15.0

5.1

8.00

2.00

2.54

20.0

15.0

5.1

8.00

2.54

Idle

1.27

20.0

15.0

2.54

8.00

1.27

2

20.0

15.0

5.1

8.00

2.00

2.54

20.0

15.0

5.1

8.00

2.54

Results

Thermal loads

Vessel heat balance

Vessel

(80 K)

(Tp)

Li2

Li1

Cryostat Tb

Qv

Qci

Li5

Coil

Qc

Tb

Li4

Ports

Shell

Qp

Qt

Qn

Li6

Qa

Qh

MODEL

15 C

Vessel

(80 K)

350 C

Cryostat Tb

3.3

2.7

Coil

8.6

Ports

Shell

9.8

0.33

16.5

1.0

5.9

150 C

BAKEOUT

15 C

Vessel

(80 K)

21 C

Cryostat Tb

3.3

1.1

Coil

3.4

Ports

Shell

4.7

0.13

4.6

0

4.7

21C

OPERATION

15 C

Heaters

Heaters

Port Losses

Model is calculated in reverse, starting at flange, adding heat.  Heaters are represented by negative input value.

Qi

To

b

Li

Le

Tb

Tc

80 K

Insulation

Port

Qj

Results

Bakeout

Based on a 8” diameter port, 1 meter long. Heat input to the port is 65 watts.

Operation

Based on a 8” port, 1 meter long.  Heat input to the port is 54 watts.

Heat applied over outer half of 8”port.  Heat input to the port is 33 watts.

Port flg

Heat applied at midpoint of 8”port.  Heat input to the port is 58 watts.

Summary

During bakeout the vessel tracing will be required to supply on the order of 16.5 kW of heating to the vessel wall.  The port electrical heaters will have to supply on the order of 5.9 kW.  During idle[preshot] operation these values drop to 4.6 kW and 4.7 kW respectively.

The port extensions need to have heat distributed evenly along their length, particularily during operation.  Models representing heat added at the midway point and along the outer half of the extensions had unacceptable temperature gradients, resulting in portions of the pipe running well below room temperature.  Attempts to boost the temperatures up to room temperature resulted in very high temperatures at the port flanges.  This will make replacement of heaters very difficult or impossible since there is no access to the entire length of the extension once the vessel is assembled.  The recommended solution is to add redundant heaters to the ports.  An internal heater could be added inside the port on the vacuum side.in the event both heaters are lost.

The outside of the cryostat will run cold and result in icing unless a nominal ammount of heat is suupied to its surface [ on the order of 2.5 kW].  This could be circulating fans or resistance heaters.

The cryostat gaseous nitrogen system will require 13.4 kW of cooling during bakeout and 8.1 kW during idle operation.

The modular coil liquid nitrogen system will require 11.3 kW of cooling during bakeout and 4.5 kW during idle operation.

Caveat:  This analysis is strictly a global overview of the heat balance mechanisms at work in the vessel system; it is based on simple area/conduction relationships assuming perfect contact between components such as the modular coils and vessel and ignores details of the local geometry.  It is meant as a first cut for design purposes and is useful for bounding the insulation requirements, power requirements, and tracing coolant parameters.  It will be followed up by 3-D finite NASTRAN models which will more closely represent the actual geometry.

Ports have been added to the vessel since this analysis and the total heat input to the ports will increase; total heat input to the vessel wall will decrease.  The net energy input should not change markedly.