NCSX

“Modular Coil VPI Autoclave”

FAILURE MODES AND EFFECTS ANALYSIS

(FMEA)

Prepared By:

S. Raftopoulos, Cognizant Engineer for Modular Coil Tooling

Concur:

J. Chrzanowski, Cognizant Engineer for Modular Coil Winding & Assembly (WBS 142)

Concur:

Jerry Levine, PPPL Safety

Approved By:

Wayne Reiersen, NCSX Engineering Manager

- Description of Autoclave system

  - An autoclave is a pressure chamber used to cure materials at elevated temperatures and pressures, and/or vacuum during the process and cure cycles.  The application and sequence of heating, cooling, pressure and vacuum are predetermined by the process specifications that detail the fabrication and treatment of parts.

The NCSX Modular Coil Autoclave has been specifically designed to accommodate the NCSX Modular Coils, and to provide the necessary operational parameters for the Vacuum Pressure Impregnation (VPI) process that is employed in the production of these coils.

  - Vacuum/Pressure requirements:

    - Pumpdown a base pressure equal to or less than 1 torr.

    - Pumpdown to base pressure in 4 hours or less.

    - Rate of rise (empty chamber, no pumping) to be less than 5 torr per hour.

    - Capable of withstanding 30 psia (15 psig) internal pressure.

    - Service ports

  - Heating requirements:

    - Circulating fluid (gaseous) heating system than can heat the modular coil sections to 50 degrees centigrade in 15-hour period.

    - Wall heating (resistive) capable of maintaining inner temperature (under vacuum) at 50+/-2 degrees centigrade.

    - Outer wall insulation.

    - Provide heated air for venting from vacuum condition.

  - Instrumentation and Control

    - Temperature readouts of:

      - Modular coil

      - Autoclave walls

      - Internal air/fluid

    - Vacuum readout.

    - Feedback control of wall heating system

    - Feedback control of air circulating heating system

- Operation (reference)

A typical cycle for the impregnation and curing of the modular coil conductor is as follows:

    - Remove autoclave lid, lower (wound) modular coil section into chamber and secure.

    - Connect epoxy sprus and thermocouple leads.

    - Install lid and secure.

    - Start vessel-heating system.  Heat at a rate not greater than 10 degree centigrade per hour to a temperature of 40 C.

    - Initiate wall-heating mode to maintain coil temperature at 40 C while under vacuum.

    - Evacuate autoclave to a pressure of 0.5 to 1 torr

    - Initiate epoxy impregnation by directing flow to lower spru(s).

    - Observe through viewports as impregnation process proceeds.  Valve in additional sprus as necessary.

    - Increase epoxy delivery pressure, as well as autoclave internal pressure correspondingly, as necessary to enhance epoxy flow.

    - Continue impregnation process until epoxy flows from top (exhaust) sprus.   Valve off epoxy when coil is fully impregnated.

    - Vent autoclave with 40 degree C air to 0 psig.

    - Initiate heating system and heat at a rate not greater than 5 degree centigrade per hour to a temperature of 110 C.

    - Hold at 110 degree C for 5 hours.

    - Raise temperature to 125 degree C.  Hold for 16 hours.

    - Turn off heating, and allow modular coil to cool.

Blower

Heating element

Valve (typ)

Pressure & Vacuum gauges (typ)

50-scfm vacuum pump

Autoclave

LN2 Cryo-trap

Relief valves (2)

Lighting

Modular coil

- Purpose/Scope:

The goal of this FMEA is to identify failures that will compromise the success of the VPI Process.  The scope of this FMEA covers that autoclave hardware and subsystems only.  Failure of the VPI Process component/hardware will be addressed separately.

A compromise of the VPI process is further defined as a condition in which one of the following occurs.  The VPI Cognizant Engineer shall ultimately decide whether or not the results of a VPI cycle are acceptable.

  - Failure of the impregnation process

    - Epoxy rich areas

    - Epoxy starved areas

    - Voids in the coil winding

  - Failure of the curing process

    - Epoxy leak from the bag-mold

    - Brittle condition after cure

    - Soft condition after cure

    - Cracks in cured winding

- Description of FMEA columns.

  - Component  - The piece of hardware or subsystem where the failure occurs.

  - Failure Mode/Cause - Description of the failure.

  - Effect on VPI process - The effect on the success of the VPI process.  The Effect on VPI Process has been further defined in the following categories:

    - None. Has no effect on the VPI process and does not have to be repaired before next cycle.

    - Negligible.  Has no effect on the VPI process, but should be repaired before next cycle.

    - Minimal.  May have an effect on the VPI process, unless a compensating provision is taken.

    - Marginal.  May or may not affect VPI Process, depending on severity and timing of failure.

    - Halt Process.  Requires the VPI to be halted.  The ramifications of halting the process are greatly dependant on the timing of the failure.

    - Other.  This is a failure that should not affect the VPI Process, however it could affect the operators’ ability to control or monitor the process.

  - Fault Detection/Isolation - This is the mechanism by which the failure will be detected

  - Compensating provisions - Describes action(s) taken to overcome the failure, typically without severely affecting the VPI Process.  These options typically compensate for the failure by adjusting an operating parameter, or operating equipment in a different configuration, however, if the VPI process must be halted, there is no compensating factor

  - Remarks –additional information deemed relevant.

Project:      NCSX                             FAILURE MODES AND EFFECTS ANALYSIS                page _______ of_________

Element: Autoclave for Modular Coil VPI                      Performed By: Steve Raftopoulos                                                  Date: ______________

##### Component: ____________________________		Reviewed By: _______________________________________ Date: ______________

| Component | #### Failure Mode/Cause | Effect on VPI process | Fault Detection/Isolation | Compensating provisions | #### Remarks |
| --- | --- | --- | --- | --- | --- |
| Vacuum Vessel | Small leak in weld | Minimal | Rise in tank pressure | Run 2nd pump | Re-weld or Glyptol |
| Vacuum Vessel | Large leak in weld | Halt process | Rise in tank pressure |  | Re-weld |
| Vacuum Vessel | Small leak in O-ring | Minimal | Rise in tank pressure | Run 2nd pump | Replace later |
| Vacuum Vessel | Large leak in O-ring | Halt process | Rise in tank pressure |  |  |
| Vacuum Vessel | Failure of internal lighting | Other | Obvious failure | Use external lights at viewports | Unlikely all units fail simultaneously |
| Vacuum Vessel | Failure of the pressure relief valve | Minimal | High pressure reading | 1:Redundant relief valve 2: Procedural control |  |
| Vacuum Vessel | Window failure | Halt process | Obvious failure |  | Very, very unlikely |
| Vacuum system | Failure of vacuum pump | Minimal | Rise in tank pressure | Operate backup pump |  |
| Vacuum system | Failure of vacuum gauge | None | Loss of data/readout | Redundant gauge | Replace later |
| Vacuum system | Failure of vacuum cryotrap | Minimal | Periodic monitoring per procedure. | Periodic monitoring per procedure. Install dry coalescing filter upstream | Contamination of pump oil |
| Heating system | Failure of heating element | Negligible | None | Fix when not operating |  |
| Heating system | Failure of multiple heating elements | Marginal | Temperature rise slower than usual | Elements wired in banks. | May or may not halt VPI depends on timing & severity of loss |
| Heating system | Failure of circulating fan/blower | Marginal | Audible detection of no blower/fan | Monitor multiple T/C for temp distribution | If temp distribution is OK, continue VPI at slower T ramp-up |
| Heating system | Failure of thermocouple | Loss of data | No readout | Redundant T/Cs |  |
| Heating system | Failure of thermocouple controller (low) | Loss of heating | No readout, readout does not match redundant units | Switch to one of redundant T/Cs and controllers | Periodic monitoring per procedure. |
| Heating system | Failure of thermocouple controller (high) | T ramp up too fast | T/C data will alert operators | Operate manually or switch to alternate unit controller | Periodic monitoring per procedure. |

Project:      NCSX                             FAILURE MODES AND EFFECTS ANALYSIS                page _______ of_________

Element: Autoclave for Modular Coil VPI                      Performed By: Steve Raftopoulos                                                  Date: ______________

##### Component: ____________________________		Reviewed By: _______________________________________ Date: ______________