### Fabrication and Assembly Tolerances

The accuracy to which we can fabricate and assemble the field coils represents probably the biggest concern for field errors. It also posed a large challenge in how to assess and evaluate these inherent uncertainties beforehand.

The experience gained from the stellarator community in the construction of past machines suggested tolerances on the order of one in a thousand as installed were acceptable, though not necessarily cheaply. Applying this to NCSX with a major radius of ~1.5 m would say tolerances of +/- 1.5 mm would be required. Subsequent discussions with potential manufacturers and construction groups have given us confidence that this is achievable. The question remained whether this is adequate from a field error viewpoint.

To explore the impact of coil tolerances or more generally, the impact of geometric changes to the coil windings, a large number of potential coil distortions were examined using the methods described in Section Error: Reference source not found.

First, to try and reflect fabrication tolerances, systematic perturbations were applied to each degree of freedom describing the coil geometry. This was done for both the individual coil types (i.e. modular coil types A, B and C; TF 1, 2 and 3; and PF 1 through PF 6) and the coil systems collectively. The perturbations were sinusoidal variations (where the mode number and phase of the variation were also varied) in r,  and z. A coil set containing the perturbed geometry has combined with a coil set of opposite current of the unperturbed geometry, resulting in a coil set which provided only the differential field (i.e. error field) which could be evaluated against the VMEC fixed or free boundary equilibrium background field. For each geometric perturbation applied, an evaluation of the magnetic island size induced at each (significant) resonant surface in the plasma was made. Results of this are contained in the figures which follow, taken from earlier presentations. Results are for each perturbation taken alone, where the magnitude of the perturbation is the full tolerance. A large number of cases where examined to cover the different coils and groups of coils, degree of freedom, mode and phase of perturbation.

Second, to try and reflect assembly tolerances, again systematic perturbations were applied to each degree of freedom describing the coil position and orientation (i.e. free body displacements). Again the effect on individual coil types and coil systems collectively were explored. The degree of freedom changes where done relative to a local coordinate system at the center of gravity of the coil. Rotation magnitudes were chosen to limit the maximum displacement at the coil to the specified tolerance. (Note: Some of the initial work contained herein reflected an earlier 2.0 mm tolerance instead the present 1.5 mm)

Examination of the impact of these various individual perturbations showed significant variation in impact on island size.

To try to assess how these different perturbations from fabrication and assembly might combine, a method was devised to combine them in a random fashion. A random factor was applied to coordinate change resulting from each combination of different coils and groups of coils, degree of freedom, mode and phase of perturbation. The individual coordinate changes were then summed and the resultant coordinate changes (now effectively a random function) normalized to the 1.5 mm tolerance specified. (This assumes the stack up of tolerances from all sources will be such as to assure the final location of every point in every coil is within +/- 1.5 mm) A large number of random functions were examined and the distribution of island sizes observed. Results are plotted and tabulated in the figures that follow.

What should be clear is that even at these tight tolerances, the islands produced from either systematic or random distributions of coil geometry errors are potentially damaging, possibly exceeding 20% of total flux in plasma. This would be unacceptable without some form of mitigation.

A set of in-vessel trim coils was previously designed to handle symmetry-preserving corrections, targeting the 3/5 (m=5) and 3/6 (m=6) resonances. Another set of ex-vessel trim coils was introduced to target lower order, non-symmetric resonances (1/2, 2/4, 2/3, etc). These are pictured in the figures that follow.

To demonstrate their effectiveness in island mitigation, a number of the more severe cases of islands induced from coil geometric perturbations were examined. For each case, currents in each of the trim or correction coils need to be solved for to attempt to suppress the islands without undue damage to the plasma boundary or exciting other resonances. A coupling matrix (A) was calculated which related unit currents in each of the trim/correction coils to impact on the resonant field component for each resonance.  A target vector (b) was formed of the resonances induced by the coil geometry perturbation that we are trying to suppress. The trim/correction coil current vector (x) is obtained by solving Ax=b using a SVD (single value decomposition) algorithm. The TraceBrtp code described earlier was used to visualize the field structure before and after the applied correction. Results, shown in the figures that follow, indicate even for the worse case stack ups at 1.5 mm tolerance, the total flux lost to islands can be reduced to an acceptable level without undue perturbations to the plasma boundary with acceptable current levels in the trim coils.

Not content to leave well enough alone, we tried to answer the question can we soften the tolerances (and simplify – i.e. reduce cost of – fabricating the coils). This question was examined by re-doing the random function studies using varying the tolerance values for the each of the coil systems (Modular, TF and PF) and within the Modular Coils. It was shown that tolerance control on the modular coils is most critical for regions of the coils that are in close proximity to the plasma, but could possibly be relaxed in regions far from the plasma. It was also shown that the tolerance of the TF and PF could be relaxed without significantly impacting field errors. Again, keep in mind that in all cases, the field errors would be intolerable without the use of trim coils. The real question is how good are the trim coils, in terms of how large a field error they can correct without damaging or otherwise altering the plasma configuration.

Other issues addressed herein include investigations into other forms of geometric perturbations, in addition to sinusoidal and random Fourier functional distributions. This includes local perturbations (i.e. ‘wavelet’ type) and broad deformations (i.e. large regions of the coils perturbed in the same fashion).