## Ferromagnetic Material

### Neutral Beam Magnets

The Neutral Beams being re-used from PBX contain at large bending magnetic wound around a ferromagnetic core to concentrate the field used to control non-neutrals on the beam. This posed a concern from two standpoints:

1) Does the stray field from the bending magnetic produce significant field errors at the plasma?

2) Does the remote field from the Stellarator Core magnetics further magnetize the ferromagnetic material at the NB significantly contributing to field errors at the plasma?

To answer these questions, an ANSYS model of the Neutral Beam Bending Magnet was constructed and run by Chang Jun. The self-field from the bending magnetics and ferromagnetic core during Neutral Beam Operation was evaluated at discrete points on the resonant surfaces in the plasma. Using the VACISLD Code, the resonant component of the normal field on the surface was evaluated and the island size calculated. Similarly, the field from the ferromagnetic core subjected to a background field from the stellarator core magnetics was evaluated at the plasma. This was done with and without the NB operating.

The resultant field errors were found to be less than 1 gauss. Predicted flux lost to islands was dominated by the m=2 mode with islands of 1.0%.