| NATIONAL COMPACT STELLARATOR PROJECT Engineering Change Proposal (ECP) |  |  |  |  |
| --- | --- | --- | --- | --- |
| COVER PAGE (TO BE COMPLETED BY SYSTEMS ENGINEERING SUPPORT MANAGER) |  |  |  |  |
| Originator: David Williamson |  |  | Date: June 24, 2004 |  |
| ECP No: ECP-04-010 |  | ECP Title: Prototype Winding Form Poloidal Break Geometry |  |  |
| Required Reviewers |  |  |  |  |
| Required Reviewers for this ECP: P. Heitzenroeder |  |  |  |  |
| ECP Approval Level Expedited ECP? Yes No Change Level: Approving Official: |  |  |  |  |
| Actions - Update CSPEC NCSX-CSPEC-141-01-04 - Update drawing SE141-116P to reflect approved changes |  |  |  |  |
| APPROVALS (TO BE COMPLETED BY APPROVING OFFICIALS) |  |  |  |  |
| Change Level | Approving Official |  | Approval? | Signature |
| 3 | NCSX Project Manager |  | Yes No |  |
| 3a (Expedited ECP) | NCSX Engineering Manager |  | Yes No |  |
| 2 | NCSX Federal Project Director |  | Yes No |  |
| 1 | Associate Director OFES |  | Yes No |  |
| 0 | Under Secretary of Energy |  | Yes No |  |

| NATIONAL COMPACT STELLARATOR PROJECT Engineering Change Proposal (ECP) |  |
| --- | --- |
| PART I (TO BE COMPLETED BY ORIGINATOR) |  |
| Originator: David Williamson | Date: |
| Overview of Change Type of ECP: EXPEDITED STANDARD Type of Change: TECHNICAL COST SCHEDULE EDITORIAL (Check all that Apply) Reason for Change: Vendor recommends 2.25-in cut through entire winding form section as a means of simplifying the shim geometry and improving fit-up of the shim, insulators, and bolts. Impacted WBS Elements: WBS 141 Modular Coil Winding Form Impacts of Change (Briefly Describe): This change will: - Improve the shim and installation fit-up - Remove the jog in the insulation path - Will result in an increased width of the cut at the tee web (plasma side) - Will not have a cost or schedule impact. Assessment of Other Options: None Detailed Description of the Change: (Use Continuation Sheets and/or Attach Information/Sketches, As Needed) List Attachments, Impacted Documents, etc. Drawing SE141-116P, Rev-1 incorporates these changes. Description of Change: - SE141-116P, Sheet 1, Quad B2 – changed weight from 5575-lbs to 5104-lbs - SE141-116P, Sheet 1, Quad A2 – Added typed name/date to drawing approval column - SE141-116P, Sheet 7, Quad C5 – changed 0.75-in cut to 2.25-in cut thru - SE141-116P, Sheet 7, Quad D2 – added Section C-C - SE141-116P, Sheet 7, Quad C7– changed 90-deg to basic dimension - SE141-116P, Sheet 7, Quad C3– changed 15-deg to basic dimension |  |