| NCSX RFD | Number: NCSX-RFD-00? |  | RFD Description: NCSX Vessel port manufacture |
| --- | --- | --- | --- |
| Initiator: Doug McCorkle |  | Organization: Major Tool and Machine |  |
| List of Impacted Documents: SE120-004 |  |  |  |
| Cost Impact: (If none, so state) NONE |  |  |  |
| Schedule Impact: (If none, so state) NONE |  |  |  |
| Impact on Interfaces with Other WBS Elements/Items: (If none, so state) None |  |  |  |
| Full Description of the Deviation Requested: (Use continuation pages, e-mails, letter, sketches, etc. as needed and include amplifying information as appropriate) DEVIATION: The replacement (Trinos 480FBL080-E (4.63 Blank)) flanges have clearance holes instead of design specified MDC 130022 tapped 4.63 OD Weld Flanges. This will require longer bolts and the use of nuts to attach to the port flange. Request using MDC P/N 190063 bolts, and 190180 nut plates to attach the blank flange to the Trinos flange. The magnetic permeability characteristics of the MDC nut plates are an unknown to MTM. Or, it may be possible to machine and install special inserts and use specified hardware or re-machine both flanges (weld flange and cover) and use larger diameter bolts. Request PPPL provide specific part number / part description / design specifics if one of these options are selected. Obviously, magnetic permeability would be a concern. JUSTIFICATON: Parts cannot be assembled based on current conditions. Field assembly / disassembly access may also be a concern. Cog Engineer, Paul Goranson DISPOSITION: From: Paul Goranson Choice 1. Use holes as is and MDC bolt hardware kit 190045. |  |  |  |
| Attachments: N/A |  |  |  |
| Initiator Signature: Date: |  |  |  |
| Project Disposition (Include ECP Number): |  |  |  |