| NCSX RFD Part I | Number: 12-022 |  | RFD Description: Replacement of Scrapped VVSA #3 Flanges on spacer assembly |
| --- | --- | --- | --- |
| Initiator: M. Viola |  | Organization: PPPL |  |
| List of Impacted Documents: (Specification, MIT/QA Plan, SOW, drawing, etc.) NEED DRAWING NUMBERS se121-014 |  |  |  |
| Cost Impact: (If none, so state: NONE) |  |  |  |
| Schedule Impact: (If none, so state: NONE |  |  |  |
| Quality Impact: (If none, so state): NONE |  |  |  |
| State Requirement Deviation is Requested For: (Specification, MIT/QA Plan, SOW, drawing, etc.) : NEED DRAWING NUMBERS se121-014 |  |  |  |
| Full Description of the Deviation Requested: (Use continuation pages, e-mails, letter, sketches, etc. as needed and include amplifying information as appropriate to support deviation request.) MTM advised that two flanges of one spacer needs to be re-made (the originals were cut wrong and had to be scrapped). MTM advises that they must replace the originally purchased flanges with new flanges that are 2” stock thick for VVSA #3 spacer. The replacement plate of Inconel is 2” thick. NCSX requests that MTM machine the new replacement flanges to a flat and parallel condition (1.25” vs. the original thickness of 1.00”) . With this change, the overall face to face distance will be 7” vs. the original 6.5”. |  |  |  |
| Attachments: |  |  |  |
| Initiator Signature: Mike Viola |  |  |  |

| NCSX RFD Part II | Number: 12-022 |  | RFD Description: Replacement of Scrapped VVSA #3 Flanges |
| --- | --- | --- | --- |
| RLM: Brad Nelson |  | Organization: ORNL |  |
| Impact on Interfaces with Other WBS Elements/Items: (If none, so state : NONE) |  |  |  |
| RLM Recommendation: Approve Do Not Approve Additional remarks: Should the impacted drawings be formally revised or should the “stamp” process outlined in NCSX Procedure PROC-007 be utilized? Formal revision required (Note if > 3 RFDs, then a formal revision via ECN is required) “Stamp” process outlined in PROC-007 is authorized. Does this Change Impact Material Already Procured or Parts/Assemblies Already Assembled/Manufactured using this Material: Yes No If “Yes”, what is the recommended disposition of this material/part/assembly? Scrape flanges that were incorrectly cut for VVSA #3 and replace. |  |  |  |
| RLM Signature: |  |  |  |
| Project Disposition: Approved. No ECP required. NCSX Systems Engineering Support Manager Approved. ECP - assigned and processed. Not Approved. Reason(s) for disapproval: |  |  |  |