| NCSX RFD Part I | Number: 14-004 |  | RFD Description: Add fillets to Modular Coil Lower Lead Blocks to aid machining |
| --- | --- | --- | --- |
| Initiator: John Puhl |  | Organization: JP Pattern |  |
| List of Impacted Documents: (Specification, MIT/QA Plan, SOW, drawing, etc.) Drawing: SE142C-134-r0 Drawing: SE142C-135-r0 |  |  |  |
| Cost Impact: (If none, so state) NONE |  |  |  |
| Schedule Impact: (If none, so state) NONE |  |  |  |
| Quality Impact: (If none, so state) NONE |  |  |  |
| State Requirement Deviation is Requested For: (Specification, MIT/QA Plan, SOW, drawing, etc.) Drawing: SE142C-134-r0 Drawing: SE142C-135-r0 |  |  |  |
| Full Description of the Deviation Requested: (Use continuation pages, e-mails, letter, sketches, etc. as needed and include amplifying information as appropriate to support deviation request.) The following changes are requested to to aid in machining (see attached sketches). - C02a_fillet-n-rnds_se142c-134-r0 - 1/16 fillet added to posts due to surface not being planar. Is this acceptable? If not area could be filed out.(red) - 1/16 fillet added to inner rib.(red) - 1/64 round added due to area being extremely thin. (Cyan) - 1/32 round added to ribs due to area being thin also (yellow) - C02a_fillet-n-rnds_se142c-135-r0 - 1/16 fillet added to posts due to surface not being planar. Is this acceptable? If not area could be filed out.(red) - 1/64 round added due to area being extremely thin. (Cyan) |  |  |  |
| Attachments: [1] C02a_fillet-n-rnds_se142c-134-r0.gif [2] C02a_fillet-n-rnds_se142c-135-r0.gif [3] E-mail dated July 12, 2005 from James Woznicki (JPP) to Dudek and Nelson |  |  |  |
| Initiator Signature: See Attachment [3] Date: July 20, 2005 |  |  |  |

| NCSX RFD Part II | Number: 14-004 |  | RFD Description: Add fillets to Modular Coil Lower Lead Blocks to aid machining |
| --- | --- | --- | --- |
| RLM: Brad Nelson |  | Organization: ORNL |  |
| Impact on Interfaces with Other WBS Elements/Items: (If none, so state) None |  |  |  |
| RLM Recommended Disposition: Approve Do Not Approve (If recommendation is to approve, ECP will be assigned) Additional remarks: WBS Manager (D. Williamson) – PTR (L. Dudek) – RLM Disposition: Approve with caveat that mating parts be chamfered to clear added fillets. |  |  |  |
| RLM Signature: |  |  |  |
| Project Disposition: (Include ECP Number): Approve with caveat that mating parts be chamfered to clear added fillets. |  |  |  |

Attachment [3]

From: James Woznicki [mailto:jimw@jppattern.com]

Sent: Tuesday, July 12, 2005 8:00 AM

To: Nelson, Brad E.; ldudek@pppl.gov

Cc: Johnp@jppattern.com; Jobfile Notes

Subject: NcSX Lead Supports Fillets-n-Rnds (JP20051092)

Brad/Lawrence,

I have reviewed the lower lead support data for preparation

of machining. Attached are a few images with fillets and rounds added to

aid in machining.

*	    C02a_fillet-n-rnds_se142c-134-r0

*	1/16 fillet added to posts due to surface not being

planar. Is this acceptable? If not area could be filed out.(red)

*	1/16 fillet added to inner rib.(red)

*	1/64 round added due to area being extremely thin.

(Cyan)

*	1/32 round added to ribs due to area being thin also

(yellow)

*	    C02a_fillet-n-rnds_se142c-135-r0

*	1/16 fillet added to posts due to surface not being

planar. Is this acceptable? If not area could be filed out.(red)

*	1/64 round added due to area being extremely thin.

(Cyan)

Please review images and reply if these added fillets and rounds are

acceptable.

Thanks,

James C. Woznicki

J.P.Pattern Inc.

262-781-2040 Ext 19.