Action Items for Engineering

#4	Estimate ripple effects on magnetic diagnostics and equilibrium field perturbations.

#6	First plasma metrics have been removed from the GRD. They should be picked up in the ISTP.

#10	Follow up with appropriate PPPL Departments to secure agreements on external interfaces and associated responsibilities.

#18	Assess power supply implications of loop voltage requirements (3V, upgradeable to 5V).

#20	Disposition. (Streamline requirements at the back)

#26	What analyses are needed to implement new seismic requirements?

#29	Same as  #18.

#30, 31  Metrology requirement needs implementation plan.

#32	Assess implications of new requirement on lifetime number of bakeout cycles.

#44	Review proposed disposition.

#45	Assess implications of requirement for 1000V bias on first wall sections.

#48	Assess implications of new requirement to position the machine high enough that eddy currents in the ground plane are not a problem.

#49	Disposition. (Update the Requirements Allocation Matrix and Requirements Verification Matrix.)

#50	Assess implications of new requirement on vacuum vessel weight-carrying capacity.

Action Items for Physics

#16	Determine the rise time for beta in the 1.2 T scenario.

#47, 51  Assess implications of changes in the reference scenarios.

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 1____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Page 9, Requirements Para c) “Longest lived eddy current in conducting structures shall be <20ms”. Does this mean all structures in the Test Cell? How far out? Electrical breaks? Originator L. Dudek_______________ Name/Organization ____F.O.M.________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Clarified. The 20 ms requirements applies to everything outside the vacuum vessel and inside the cryostat. The electrical breaks requirement applies to everything inside the cryostat except the vacuum vessel and coils. The cryostat boundary is chosen for simplicity. Affects 3.2.1.5.2 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. Schmidt Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 2____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Page 20 para 3.3.1.1 Magnetic Permeability <1.02 to how far out from machine? Originator L.Dudek_________ Name/Organization __F.O.M._____________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Everything inside the cryostat. Affects 3.3.1.1. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 3____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Clarify that radial position reqt does not imply a reqt for PRC to handle 16 cm ……?….. Originator R. Hawryluk Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Added clarification. Affects 3.2.1.5.3.4.5. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 4____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Define power supply coil current ripple. Originator R. Hawryluk Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur that there needs to be a limit, to keep ripple from interfering with equilibrum measurements or control. Affects 3.2.1.5.3.7. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # _5___ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Global leak rate of 1 x 10-5 tl/sec is hard to meet and an increase by a factor of l 3 Originator R. Hawryluk_________ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Increase leak rate to 2105 torr-l/s. With our 2,600 l/s of pumping, that leak rate is still compatible with the base pressure requirement of 1108 torr and some margin. Affects 3.2.1.2.2.1. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 6____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation The GRD should focus on project requirements and not on first plasma requirements, which should be documented at a higher level elsewhere. The level of detail in the PEP is applicable for the high level milestone. Originator R. Hawryluk ___ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Remove them from the GRD, and capture them in the Integrated Systems Test Plan. Affects 3.1.2, 3.2.1.1, 3.2.1.2.2.1, 3.2.1.5.3.3.2, 3.2.1.5.6.1.1, 3.2.1.5.9. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # _7___ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Remove “Voltage isolation…” sentence under electrical grounding. Should RF shielding also be addressed in the Grounding Spec? Originator W. Reiersen ________ Name/Organization ___M.E.D.____________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur (H. Neilson and S. Ramakrishnan). Affects 3.3.2.1 and Deletes 3.3.2.2. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 8____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Add “unless otherwise authorized” qualification for lithium compatibility. Originator ___W.Reiersen______ Name/Organization _____M.E.D._______________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.3.1.3. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 9____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Limitations on v <1.02 needs to be specified – not everywhere in the facility. Originator W. Reiersen__________ Name/Organization ____M.E.d.___________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Everything inside the cryostat. Affects 3.3.1.1. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 10____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Review external interfaces. Originator W. Reiersen__________ Name/Organization ____M.E.d.________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Applies to 3.2.2, but does dot require a change. That section documents the assumptions NCSX is making concerning its interfaces with the PPPL facility. The project will follow up with the relevant PPPL Departments to secure agreements on these interfaces and associated responsibilities. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 11____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Review upgrades for inboard fueling, especially i.e. things w/vv interfaces. Originator ___W. Reiersen_______ Name/Organization ___M.E.D._________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Requirements were reviewed by H. Kugel. Affects 3.2.1.5.7.2 b). 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 12____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation IC: 6MW Add TBR. Compatibility w/new VV geometry needs to be determined. Originator W.Reiersen__________ Name/Organization __M.E.D.__________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.6.2. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 13____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Requirement for electrical breaks should apply to things outside the VV and in-vessel components. Time constants should not be applied outside stellarator coil. Originator W. Reiersen_______ Name/Organization _M.E.D.___________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Clarified. The 20 ms requirements applies to everything outside the vacuum vessel and inside the cryostat. The electrical breaks requirement applies to everything inside the cryostat except the vacuum vessel and coils. The cryostat boundary is chosen for simplicity. Affects 3.2.1.5.2 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 14____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Should “or” be in the field error requirement? Originator W. Reiersen__________ Name/Organization __M.E.d.__________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) It should be “and”. The implementation is to make each of the contributions individually small compared to 10% and not try to come up with a way to sum them. Affects 3.2.1.5.1. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 15____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Use PIES to confirm the VMEC plasma configurations over flexibility space. Originator W.Reiersen__________ Name/Organization _M.E.D.___________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur, but this is a management issue, does not impact the GRD. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 16____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Are the timescales for heating to fuel beta (100ms) acceptable? Originator W. Reiersen________ Name/Organization __M.E.D.______________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) It may be an issue at 1.2 T, where beta could rise more quickly. Add “TBR” to the rise time. Affects 3.2.1.5.3.3.1.5. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 17____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Add GDC to “15 min. intervals when constrained by coil or PFC cooldown.” Originator W.Reiersen_______ Name/Organization __M.E.d._________ __ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.10. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 18____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Is the loop voltage implied by the ref. Scenarios adequate to assure inductive breakdown? Originator _W.Reiersen_____ Name/Organization _M.E.D._____________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) No, there needs to be an explicit requirement for 3V, upgradeable to 5V, to cope with dirty plasma conditions. New Requirement 3.2.1.5.3.6. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 19____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Need to define scope (negotiate scope) with DOE – PDR will only cover VV and Modular Coils. (Hawryluk cognizant) Originator W.Reiersen for RH_____ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. This is a management issue, does not impact the GRD. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 20____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Blow away requirement areas that do not add value (at back end of spec). Originator W.Reiersen____ Name/Organization __M.E.D.______________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Action: Wayne. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 21____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation (5) kV isolation between V.V. and systems attached to it other grounds (diagnostics,…). Originator E. Fredrickson________ Name/Organization _PPPL___________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Retained a general requirements for voltage isolation between the vacuum vessel and attachments, but refer to the NCSX Grounding Spec for all details, including the isolation voltage. Affects 3.3.2.1. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 22.1____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation - Words should be added on maximum neutron generation, i.e., per shot, per second, per year, and/or lifetime. Originator J.Levine__________ Name/Organization __ES&H__________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Added clarification that an annual DD neutron yield of 4.61016 per year corresponds to 1 rem per year dose-equivalent in the control room. Affects 3.3.6.8. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 22.2____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation - The following sentence should be added to Section 3.3.6.7: “Designs shall comply with the requirements of ES&HD 5008, Section 2.” Originator J.Levine__________ Name/Organization __ES&H__________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.3.6.7. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 23____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Would like to reiterate that Section 4 of the GRD should be renamed from “Quality Assurance Provisions” to Verification and Validation.” QA Provisions include much more than is discussed here, or should be discussed here, including design verification, management and independent assessments, calibration. The given title is a misnomer. Originator __Judy Malsbury_____ Name/Organization ____QA________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) The text explains that this section deals with verification of requirements, so the title is changed to “Verification of Requirements.” Affects Section 4. title. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 24____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Figure 3-1 is difficult to read so can’t determine if it is accurate. What does the “OR” in the circles mean? Originator Judy Malsbury_________ Name/Organization ___QA_________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Improved readability by enlarging the words and expanding the figure to fill the page. “OR” designates a branch node. Affects Figure 3-1. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 25____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation For both NSTX and TFTR, we maintain(ed) availability statistics via manual entries made into a database by the COE (or designee). Should be consider defining requirements for determining system availability and try to automate much of this now? It would primarily impact instrumentation and control. Originator Judy Malsbury______ Name/Organization ____QA________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) This is a task for the operating phase. Does not impact GRD. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 26____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation In Section 3.3.1.6 on Seismic Criteria, add the following at the end of the Section: “for PC1 facilities.” PC1 (Performance Category 1) facilities are low hazard facilities, and is the same classification as NSTX. Originator Jerry Levine_________ Name/Organization __ES&H__________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.3.1.6. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 27____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Coupled power for ICRF system is dependent on size (plasma-facing area) of cavities incorporated into the vacuum vessel for the antennas. Cavity size (plasma facing area) is not yet finalized. ICRF power  5MW/m2 x total area in m2. Originator Richard Majeski____ Name/Organization __PPPL_______________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) OK. Change the requirement to 6 MW (TBR). Affects 3.2.1.5.6.2. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 28____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Vessel cooling should be sufficient to allow for a 250 C liner (upgrade reg. For lithium liner). Originator Richard Majeski________ Name/Organization ___PPPL_________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.4.2. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 29____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Need specification of maximum Vloop to ensure breakdown and plasma formation. Originator M. Zarnstorff_______ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Added an explicit requirement for 3V, upgradeable to 5V, to cope with dirty plasma conditions. New Requirement 3.2.1.5.3.6. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 30____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Need requirement on location of vessel relative to the coils/field. Originator M. Zarnstorff________ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) The “location of the vessel” is not well defined because it is a large structure with tolerances as large as 3/8-inch. However, fiducial markers on the vessel and coils are needed in order to be able to locate attachments with high accuracy. This is captured in a new requirement for metrology. Affects 3.3.1.7. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 31____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Need requirement for Fiducial Markers on the vessel and coils for use during installation and locating of in-vessel components. Originator M. Zarnstorff__________ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. This is captured in a new requirement for metrology. Affects 3.3.1.7. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 32____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Is there a limit on the number of bakeout cycles? Should there be a requirement on minimum number? Originator M. Zarnstorff_______ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Add new requirement for 100 bakeout cycles over the life of the machine. Based on 1 bakeout at the beginning of a run and up to 3 bakeouts during a run. Two runs per year for 10 years. 25% contingency. (1+3)x2x10x1.25 = 100. Per Hutch Neilson and Henry Kugel. New Section 3.2.1.2.3.6. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 33____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Section 3.2.1.5.1 should read: “The toroidal flux…due to fabrication errors, magnetic materials, and eddy currents shall not exceed 10% of the toroidal flux > the plasma (including compensations). Originator M. Zarnstorff_________ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.1. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 34____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Section 3.2.1.5.2 (b) should also except the vessel (in addition to the coils). Originator M. Zarnstorff________ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.2 b). 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 35____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Section 3.3.1.2 (a) and (b) should be modified as indicated. 3.3.1.2 (a) The Vacuum Vessel interior and all in-vessel metallic components shall be electro-polished prior to installation, except when explicitly authorized by the project. (b) The Vacuum Vessel and all in-vessel components shall be degreased and cleaned prior to installation. They will be vacuum baked, except as authorized by the project. Originator M. Zarnstorff Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.3.1.2. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 36____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation 3.3.1.3, second paragraph: Materials used inside the vessel shall be compatible with lithium, except as authorized by the project. Originator M. Zarnstorff_______ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.3.1.3. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # _37___ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Section 3.1.1.5.3.3.1.5: first sentence should read “the 1.2T long pulse high beta scenario…” the 100 ms time of rise of  should be TBD. The final bullet: “at least 1.1 sec. Originator M. Zarnstorff_______ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.3.3.1.5 in the current draft. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 38____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation 3.2.1.5.6.2: “6 MW (TBR) of ICH…” 3.2.1.5.6.3: “…3 MW (TBR) of ECH…” Originator M. Zarnstorff__________ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.6.2 and 3.2.1.5.6.3. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 39____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation 3.2.1.5.7.1: “… or other non-corrosive gasses.” Originator M. Zarnstorff Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.7.1. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 40____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation 3.2.1.5.7.2 – In general, the physics requirements still need to be developed. In particular, as an upgrade capability, we will need to accommodate at least  4 gas injectors per period. (Inside & outside, top & bottom divertors.) Also, the supersonic injectors may need to use inboard. Originator M. Zarnstorff_____ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.7.2 a) and b). 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

$

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 41____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Recommend minor changes to “Base Pressure” and “Pumping Speed” sections (see attached). Originator W. Blanchard______ Name/Organization ____________________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur, with the proviso that base pressure and leak rate must be specified. Affects 3.2.1.2.2.1 and 3.2.1.2.3.1 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 42____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation - Under “During Bakeout” - Under “Capable of GDC” - Change “-any of: hydrogen, deuterium, helium, methane” To read “-any of hydrogen, deuterium, helium, and other noncorrosive gases. Originator H.W. Kugel_______ Name/Organization _ NCSX/Physics___________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Incorporated by modifying 3.2.1.4.1 (GDC Between Pulses), which the GDC During Bakeout Section (3.2.1.2.3.5) cross-references. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 43____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation - Under GDC between discharges - Change “-Any of: hydrogen, deuterium, helium & methane” To “-any or: hydrogen, deuterium and other non-corrosive gases Originator H.W. Kugel__________ Name/Organization __NSTX Physics________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.4.1 b). 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 44____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation It would be helpful to have a matrix of temperature ranges for the various components, such as that which is attached (next page), possibly including the maximum time allotted to transition from one condition to another. (Note: the table has not been updated from the CDR for the new requirements.) Originator _B. Nelson______ Name/Organization ___ORNL__________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) This table should be updated to the current requirements and used by Engineering if is found convenient to do so. It may be overly prescriptive for the GRD, since some of the entries are requirements while others are free parameters. It does identify the need to set requirements on Pre-Run (standby) temperature. Affects 3.2.1.213. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 45____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation The electrical bias for the first wall does not give a voltage range. Suggest adding a voltage limit such as “up to 1000 V”. Originator B. Nelson___________ Name/Organization _____ORNL____________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.4.1. c). 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 46____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation The 30 micro-inch finish required for in-vacuum surfaces seems very restrictive considering things like all the port extension assembly welds that must be polished in place. Can the 30 micro-inch surface finish be modified to include a caveat that the requirement can be relaxed with project permission? Originator B. Nelson Name/Organization ___ORNL________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Requirement is to electropolish except as authorized by the project. Affects 3.3.1.2 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 47____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation The 350-kA (1.8 T) Ohmic Scenario should be changed to 320 kA (1.7 T) to be consistent with providing a factor 2 range of internal iota flexibility at the nominal field of 1.7 T. Originator N. Pomphrey Name/Organization ___PPPL________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Also applies to the maximum current for disruptions. Affects 3.2.1.5.3.3.1.7 and 3.2.1.5.5 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 48____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation The machine should be positioned high enough above the existing copper gound plane in the test cell such that eddy currents in the ground plane do not become a problem. Originator S. Ramakrishnan Name/Organization ___PPPL________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Added Requirement 3.2.1.5.2 f). 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 49____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation Update the requirements allocation matrix and Requirements Verification Matrix. Originator H. Neilson Name/Organization ___PPPL________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Action: Wayne. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 50____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation The GRD should contain a requirement that either specifies or ensures adequate weight-carrying capacity in the vacuum vessel and the vessel support structure. We should have an entry in the GRD specifying that the vessel and support system must mechanically support all anticipated loads (as above) including upgrades and maintain their allignment. Originator M. Zarnstorff Name/Organization ___PPPL________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Added requirement for Vacuum Vessel weight-carrying capacity. Affects 3.3.1.8. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |

| WP # (ENG-032) PPPL DESIGN REVIEW CHIT CHIT # 51____ |  |  |
| --- | --- | --- |
| COMPONENT/SUBSYSTEM/SYSTEM NCSX GRD REview Cognizant Design Engineer H. Neilson Date of Review 04-03-03 |  | PEER CDR PDR FDR |
| SUBJECT: (CHECK AS APPLICABLE) Requirements Hardware Safety Analysis Configuration Cost/Schedule Performance Reliability/Maintainability Quality |  |  |
| Comment/Concern/Recommendation The full-current, zero-beta S2 equilibrium condition leads to pathological PF coil currents in the lastest design, translating to excessive coil current swings (including polarity changes) in going from S1 to S2 to S3. A redefined S2 with 70% current, zero beta has more reasonable currents. Modify the reference scenarios accordingly. Originator H. Neilson Name/Organization ___PPPL________ |  |  |
| REVIEW BOARD COMMENT/RECOMMENDATION (Address technical, cost, and schedule impacts as appropriate. If CHIT is not adopted, provide technical reason - do not simply state “out-of-scope or N/A” without explaining.) Concur. Affects 3.2.1.5.3.3.1.3, 3.2.1.5.3.3.1.4, and 3.2.1.5.3.3.1.6. 0 Concur 0 DISAGREE 0 OTHER CHAIRPERSON J. SCHMIDT Date: |  |  |
| Cognizant Design Engineer’s Response/Disposition: Signature Date: |  |  |
| Responsible RLM Review |  |  |
| 0 APPROVE COG DISPOSITION 0 DISAPPROVE COG DISPOSITION | Signature Date: |  |
| Cognizant Design Engineer Close-Out Sign when action required by disposition is complete. Signature Date: |  |  |