## Specification

For

#### NCSX Modular Coil

# Autoclave Vessel Fabrication

### NCSX-CSPEC-142-01-00

Prepared by:													Steve Raftopoulos, Cognizant Engineer for Modular Coil Tooling

Concur:

J. Chrzanowski, Cognizant Engineer for Modular Coil Winding & Assembly, WBS 142

Concur:

D. Williamson, WBS Manager for Modular Coils, WBS 14

Concur:

Judy Malsbury, NCXS QA Representative

Approved by:

B. Nelson, Project Engineer for Stellarator Core Systems, WBS 1

Controlled Document

This is a controlled document.  Check the NCSX Procurement Web prior to use to assure that this document is current.

- SCOPE

  - This document provides the requirements and specifications for the fabrication of the NCSX Modular Coil Autoclave vessel.

  - The vessel consists of the tank, ports and the support structure.

  - Platforms, heating, vacuum and control systems are NOT covered in this specification.

- APPLICABLE DOCUMENTS

  - Fabrication Standards

    - EM-002  “PPPL General Welding & Brazing Requirements”

  - Testing Standards

    - ENG-014 "Hydrostatic and Pneumatic Testing"Rev.0

  - Drawings

    - The Autoclave vessel shall be fabricated per the following drawings:

      - SE144-307

      - SE144-308

      - SE144-310

      - SE144-312

      - SE144-313

      - SE144-316

      - SE144-307

      - SE144-331

      - SE144-332

      - SE144-335

      - SE144-340

      - SE144-341

      - SE144-344

      - SE144-345

      - SE144-363

      - SE144-371

      - SE144-378

      - SE144-379

    - Other

      - NCSX-BSPEC-142-01-00 “Autoclave Design Specification”

      - NCSX-BSPEC-142-02-00 “Autoclave Heating System Fabrication Specification”

- REQUIREMENTS:

  - System Definition

    - The autoclave vessel chamber houses materials that are cured at elevated temperatures, under pressures and/or vacuum.  The vessel is designed to operate from room temperature to 300 degrees Fahrenheit, and withstand a pressure range from full vacuum (0 psia), to one atmosphere gauge (30 psia).  The vessel consists of cylindrical mid-section, with upper and lower domes.  The lower dome is welded to the mid-section, while the upper dome is attached via a bolted flange.  An O-ring provides a vacuum/pressure compatible seal.

  - Characteristics

    - Performance

      - Vacuum capability

        - The vessel shall be capable of achieving and maintaining a base pressure of 1x10-3 torr.

        - The aggregate leak rate for the vessel (includes all welds and O-ring seals) shall be not greater that 1x10-4 torr-liters/sec., as measured with a Helium Mass Spectrometer Lead Detector (HMSLD).

      - Pressure capability

        - The vessel shall be capable of achieving and maintaining a maximum positive pressure of 15 psig (30 psia).

        - All vessel joints shall be checked for gross leaks.  Leaks shall be repaired until undetectable.

        - The vessel assembly shall have an aggregate leak rate less than 0.5 psig/hour when pressurized to 15 psig.

      - Temperature capability

        - The vessel shall be compatible with sustained operation at 300 degrees Fahrenheit, while under vacuum or at maximum pressure.

  - Design and Construction

    - Production Drawings

      - The Autoclave vessel shall be fabricated and assembled in accordance with the drawings, parts list, and other documents listed in section 2.3.1

    - Standards of Manufacture

      - Mechanical Fabrication

        - Tolerances are specified on the mechanical drawings.  Dimensional tolerances are to be met unless stated otherwise, in writing, by the Cognizant Engineer.

      - Welding

        - Welding shall be done in accordance with PPPL Procedure EM-002 “General Welding and Brazing Requirements”.

        - Weld inspections shall be identified on the part drawing.

    - Workmanship

      - In addition to adherence to PPPL procedures, best shop practices shall be employed.

        - Remove burrs and sharp edges.

        - Completed parts must be clean of dirt, grease, cutting fluids and other foreign matter.

        - Vessel internal surfaces must be free of scale.

  - Documentation

    - Maintain and forward copies to the Operations center of all material tests, certifications and inspections, accumulated during the fabrication, setup and pre-operational testing phases.

- Quality Comformance

  - The following chart delineates the actions to be taken to verify the requirements listed in section 3

| Requirement | Description | Verification | Remarks |
| --- | --- | --- | --- |
| 3.2.1.1 | Vacuum |  |  |
| 3.2.1.1.1 | Achieve base pess. | Test | Pump down test |
| 3.2.1.1.2 | Leak rate | Test | With HMSLD |
| 3.2.1.2 | Pressure |  |  |
| 3.2.1.2.1 | Hold pressure | Test |  |
| 3.2.1.2.2 | Gross leaks | Test | Soap bubble test |
| 3.2.1.2.3 | Leak down rate | Test | Less that 0.5 psig/hour@15 psig |
| 3.2.1.2 | Temperature |  |  |
| 3.2.1.2.1 | Temp readouts | Test | ISTP |
| 3.3.2 | Manufacture |  |  |
| 3.3.2.1 | Mechanical | Inspection | Conformance to drawings |
| 3.3.2.2 | Welding | Inspection | Per EM-002 |
| 3.3.3 | Workmanship |  |  |
| All | All | Inspection |  |